28. ADC

#include <LPC214x.h>

#define LEDS   (0xFF << 8)     // P0.8 – P0.15

#define AD0_1  (1 << 24)       // P0.28 as AD0.1

#define CLKDIV (1 << 8)

#define PDN    (1 << 21)

#define BURST  (1 << 16)

void delay(unsigned int d)

{

unsigned int i, j;

for(j = 0; j < d; j++)

for(i = 0; i < 800; i++);

}

void adc_init(void)

{

AD0CR = (1 << 1) | CLKDIV | PDN | BURST;   // AD0.1, burst mode

}

unsigned int adc_read(void)

{

unsigned int val;

val = AD0DR1;           // Read ADC data register

val = (val >> 6) & 0x3FF; // Extract 10-bit result

return val;

}

int main(void)

{

unsigned int adc_val;

IODIR0 |= LEDS;          // LEDs as output

PINSEL0 = 0x00000000;

PINSEL1 |= AD0_1;        // Enable AD0.1 pin

adc_init();

while(1)

{

adc_val = adc_read();

adc_val >>= 2;       // Convert 10-bit to 8-bit

IOSET0 = LEDS;       // Turn OFF LEDs

IOCLR0 = adc_val << 8; // Display ADC on LEDs

delay(1000);

}

}

27. UART

#include <LPC214x.h>

void UART0_Init(void)

{

VPBDIV = 0x01;          // PCLK = 12 MHz

PINSEL0 |= 0x00000005;  // P0.0=TXD0, P0.1=RXD0

U0LCR = 0x83;           // 8-bit, no parity, 1 stop, DLAB=1

U0DLL = 0x27;           // 19200 baud @ 12 MHz

U0DLM = 0x00;

U0LCR = 0x03;           // DLAB = 0

U0FCR = 0x01;           // Enable FIFO

}

void UART0_Send(unsigned char ch)

{

while(!(U0LSR & 0x20)); // Wait for THR empty

U0THR = ch;

}

int main(void)

{

unsigned char data;

UART0_Init();

while(1)

{

if(U0LSR & 0x01)    // Data received

{

data = U0RBR;  // Read received data

UART0_Send(data); // Echo back

}

}

}

26.7 SEGMENT

#include <LPC214x.h>

#define DS3   (1 << 13)          // Display select

#define DS4   (1 << 12)

#define SEG   (0xFF << 16)       // P0.16–P0.23

unsigned char seg_dat[10] =

{0x3F,0x06,0x5B,0x4F,0x66,0x6D,0x7D,0x07,0x7F,0x67};

void delayms(unsigned int d)

{

unsigned int i, j;

for(i = 0; i < d; i++)

for(j = 0; j < 1007; j++);   // for 12 MHz

}

int main(void)

{

unsigned char count = 0;

PINSEL0 = 0;

PINSEL1 = 0;

IODIR0 |= SEG | DS3 | DS4;

IOSET0 = SEG | DS3;   // Disable DS3

IOCLR0 = DS4;         // Enable DS4

while(1)

{

IOCLR0 = SEG;

IOSET0 = seg_dat[count] << 16;

delayms(1000);

count++;

if(count > 9) count = 0;

}

}

25.READ SWITCH DISPLAY LED

#include <LPC214x.h>

#define LEDS (0xFF << 8)   // P0.8 – P0.15

int main(void)

{

unsigned int sw;

PINSEL0 = 0x00000000;   // Port0 as GPIO

IODIR0 |= LEDS;         // LEDs as output

while(1)

{

sw = IOPIN0 & LEDS; // Read switch status

IOSET0 = LEDS;     // Turn OFF all LEDs

IOCLR0 = sw;       // Turn ON LEDs based on switch

}

}

24.BLINKING LED USING DELAY

#include <LPC214x.h>

#define LEDS (0xFF << 8)   // P0.8 – P0.15

void delay(unsigned int d)

{

unsigned int i, j;

for(j = 0; j < d; j++)

for(i = 0; i < 800; i++);

}

int main(void)

{

PINSEL0 = 0x00000000;

IODIR0 |= LEDS;

while(1)

{

IOSET0 = LEDS;

delay(1000);

IOCLR0 = LEDS;     // LEDs OFF

delay(1000);

}

}

7 SEGMENT

int buttonPin = 12;

int delay_time = 200;

int digit = 0;

/* Segment pins a–g connected to 1–7 */

int segPins[7] = {1, 2, 3, 4, 5, 6, 7};

/* Common cathode 7-segment codes (a–g) */

byte segCode[10][7] = {

{1,1,1,1,1,1,0}, // 0

{0,1,1,0,0,0,0}, // 1

{1,1,0,1,1,0,1}, // 2

{1,1,1,1,0,0,1}, // 3

{0,1,1,0,0,1,1}, // 4

{1,0,1,1,0,1,1}, // 5

{1,0,1,1,1,1,1}, // 6

{1,1,1,0,0,0,0}, // 7

{1,1,1,1,1,1,1}, // 8

{1,1,1,1,0,1,1}  // 9

};

void setup()

{

pinMode(buttonPin, INPUT_PULLUP);

for(int i = 0; i < 7; i++)

pinMode(segPins[i], OUTPUT);

}

void loop()

{

if(digitalRead(buttonPin) == LOW)

{

showDigit(digit);

digit++;

if(digit > 9) digit = 0;

delay(300);  // debounce

}

}

void showDigit(int num)

{

for(int i = 0; i < 7; i++)

digitalWrite(segPins[i], segCode[num][i]);

delay(delay_time);

}

HELLO WORLD

#include <reg51.h> #include <stdio.h>

void main (void)

{

SCON = 0x50; /* SCON: mode 1, 8-bit UART, enable rcvr */ TMOD = 0x20;

/* TMOD: timer 1, mode 2, 8-bit reload */ TH1 = 0xFD; /* TH1: reload

value for 9600 baudrate */

TR1 = 1; /* TR1: timer 1 run */

TI = 1; /* TI: set TI to send first char of UART */ while (1)

{

printf ("Hello World ! \n ");

}

}